PE1 , PE2 and PE3 are connected to south leds .

PE1 is connected to red .

PE2 is connected to yellow .

PE3 is connected to green .

(SOUTH PE1-3)

PD1 , PD2 and PD3 are connected to west leds .

PD1 is connected to red .

PD2 is connected to yellow .

PD3 is connected to green .

(WEST PD1-3)

PE4 and PE5 are connected to walking leds .

PE4 is connected to red .

PE5 is connected to green .

States (r y g) (r y g) (r g)

Note that the format of the leds is :

South Leds (GREEN  YELLOW RED) - West Leds (GREEN  YELLOW RED) - Walking Leds (GREEN RED)

South/walking leds    west leds

Cars Going South     :  goS		                        (01_100_0)   (001_0)

Stop South Cars	    :  waitS	      	          (01_010_0)   (001_0)

Cars Going West     :  goW		         (01_001_0)    (100_0)

Stop West Cars        : waitW	                       (01_001_0)    (010_0)

Walk                         : walk		                       (10_001_0)    (001_0)

WalkFast		  :  walkFast                               (10_001_0)    (001_0)

Lights All Lamps to Red   : allRed                         (01_001_0)    (001_0)

States according to Inputs

000=No padestrian/No cars

001= No padestrian/South car

010=No pedetrian/West car

011= No pedetrian/South&west car

100=Pedetrian/No cars

101=Pedestrian/South car

110=Pedestrian/West car

111=Pedestrian/South&west cars

To need creating one to one mapping for all states.

I had to think  all diagnostic conditions .

Transition graph :

| Inputs : --------- | 000 | 001 (s) | 010 (west) | 011 | 100 (walk) | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| goS | waitS | goS | waitS | waitS | WaitS | waitS | waitS | waitS |
| waitS | goW | goS | goW | goW | allRed | allRed | allRed | allRed |
| goW | waitW | waitW | goW | waitW | waitW | waitW | waitW | waitW |
| waitW | allRed | goS | goW | goS | allRed | allRed | allRed | goS |
| walk | walkFast | walkFast | walkFast | walkFast | walk | walkFast | walkFast | walkFast |
| walkFast | goS | goS | goW | goS | walk | goS | goW | goW |
| allRed | walk | goS | goW | goW | walk | walk | walk | walk |